# Return the Tide

Resource Handbook

APPENDICES

Appendix A

GLOSSARY OF

TECHNICAL TERMS

This glossary provides non-technical definitions of technical terms, some of which are used in this manual.  This is by no means an exhaustive list of all the terminology pertaining to tidal marshes.  For more detailed reference to tidal marsh terminology, see the references listed in Section 9 of this manual.

| accretion | the gradual build up of surface elevations due to the deposition of suspended sediments on the marsh surface |
| --- | --- |
| adsorb | the chemical adhesion of one substance to the surface of another (e.g., nutrients and pollutants may be adsorbed to the surface of sediment particles) |
| aquatic | in or near water in such habitats as ponds, lakes, rivers and oceans |
| avian | relating to birds |
| back-barrier marsh | a marsh that forms in the low-lying area behind a barrier beach formation |
| barrier beach | an elongated landform created by the deposition of sedimentary materials by wind and wave currents, usually parallel to the shoreline, with water on at least two sides, and composed of sand, gravel, or cobblestones |
| benthic | relating to or occurring on the bottom of a body of water |
| best management practice | design or construction standards that are recommended to minimize the impact of development on the environment |
| biota | living organisms (plant, animals, etc.) in an ecosystem |
| brackish marsh | tidal marshes where the average water salinity is less than 18 parts per thousand (ppt) but greater than 0.5 ppt which is the upper limit of salinity in a freshwater tidal wetland |
| buffer zone | an undeveloped area bordering on a wetland that serves to lessen the impact of disturbance (e.g., urban development) |
| carrying capacity | the population of a species that an area can support without deterioration |
| cobble | a stone larger than a pebble but smaller than a boulder (2.5 to 10 inches in diameter |
| deepwater habitats | permanently flooded areas deeper than 6.6 feet (e.g., lakes) |
| degraded | characterized by loss of natural ecological structure or function |
| detrital food chain | food chain dependent upon decomposed plant and animal material as the source of energy |
| detritus | particles that result from the disintegration of organic material |
| diurnal flooding | flooding that occurs on a daily basis |
| dominant plant community | a single species or association of plants that are indicative of the ecology of an area, (e.g., in a cattail marsh the dominant plant community is cattails) |
| drainage pattern | the paths followed by surface runoff from precipitation within a watershed |
| ecology | the study of interactions between living things and their environment |
| ecological integrity | the natural (undisturbed) quality of an ecosystem |
| ecosystem | a community of plants and animals and the physical environment they inhabit (such as estuaries and tidal wetlands) which results from the interactions among soil, climate, vegetation, and animal life |
| emergent plant | erect, rooted, herbaceous plants that can tolerate flooded soil conditions, but not prolonged periods of being completely submerged; these include grasses, sedges, rushes, and rooted aquatic plants; there are two types of emergent plants: |
|  | persistent - emergent plants whose stems remain standing through the winter until the beginning of the next growing season (i.e., they persist), (e.g., cattails or bulrushes) |
|  | non-persistent - emergent plants whose stems and leaves break down at the end of the growing season; from late fall to early spring there are no visible traces of these plants above the surface of the water (i.e., they do not persist) |
| estuary | habitats partially enclosed by land but having an opening to the ocean where saltwater from the ocean mixes with freshwater from inland rivers and surface runoff |
| exemplary community | an area selected by the Maine Natural Heritage Program as being an outstanding example of the natural plant and animals found in a particular ecosystem |
| fill | material, usually associated with the dredging of a harbor or inlet, placed on the surface of the marsh; the change in elevation caused by the disposal of this material in the marsh can lead to the loss of the area as a functioning tidal marsh |
| food chain | an arrangement of organisms of an ecological community according to the order of production and consumption; at the bottom of the food chain are photosynthetic plants and phytoplankton; zooplankton and animals that feed on plants form the next level on the food chain, while predators form the highest levels |
| formerly tidal marshes | coastal wetlands that were once connected to tidal flow but have since been isolated from tidal waters by the construction of a man-made obstruction |
| freshwater source | the point of origin of nontidal waters including rivers, streams and surface runoff |
| freshwater tidal marshes | marshes that are tidally influenced, but where the average water salinity is less than 0.5 parts per thousand |
| geomorphology | the study of the natural processes involved in the creation of landforms such as tidal marshes and barrier beaches |
| habitat | the environment in which the requirements of a specific plant or animal are met |
| heavy metal | a group of dense metals, including mercury, lead, cadmium, and others, that share the characteristic of being accumulated in organisms and tend to become increasingly concentrated in organisms higher up on the food chain |
| herbaceous plant | a non-woody plant with a soft stem (e.g., bulrushes and cattails) |
| high marsh | areas of tidal marshes that are irregularly flooded (frequently beyond the reach of daily flooding) and are typically dominated by salt hay grass (Spartina patens) |
| hydric soil | a soil that is saturated, flooded, or ponded long enough during the growing season to develop anaerobic (oxygen deficient) conditions in the upper part of the soil. Hydric soils are generally poorly drained or very poorly drained. |
|  | poorly drained - water is removed from the soil so slowly that the soil is saturated periodically during the growing season or remains wet for long periods |
|  | very poorly drained - water is removed from the soil so slowly that water remains at or near the surface during most of the growing season; this is the most dominant soil drainage class of tidal marshes |
| hydrology | the scientific study of the properties, circulation, and distribution of water as it occurs in the atmosphere and at the earth’s surface as streamflow, precipitation, soil moisture, and ground water |
| hydrologic regime | the frequency and duration of flooding and/or saturation |
| hydroperiod | the duration of typical flooding/saturation events; in tidal marshes, the hydroperiod can range from daily flooding to irregular flooding (e.g., every few days, weeks, or months); depends on the marsh elevation |
| hydrophyte | a plant that is adapted for life in water or in periodically flooded and/or saturated anaerobic (oxygen poor) soils (e.g., cattails, saltwater cordgrass) |
| intertidal emergent | an erect rooted herbaceous plant growing in the intertidal zone |
| intertidal unconsolidated bottom | wetlands that have at least 25% cover of particles smaller than stones, less than 30% vegetative coverage, and are only intermittently exposed, such as pannes and tidal creeks |
| intertidal unconsolidated shore | wetlands which have at least 75% coverage of stones, boulders or rocks, less than 30% vegetative coverage, and are alternately flooded and exposed by tides |
| intertidal zone | areas that are alternately exposed and flooded by tides |
| invasive species | plant species that, when introduced to an ecosystem, can disturb the natural balance and habitat diversity by invading and dominating the natural tidal marsh plant community, frequently establishing dense monotypical (single species) stands of vegetation |
| low marsh | areas of marsh that are flooded twice a day and are dominated by saltwater cordgrass (Spartina alterniflora) |
| marine | relating to ocean environments |
| marsh hydrology | this term describes (1) the hydrologic pathways such as precipitation, surface runoff, ground water, tidal fluctuations and flooding rivers which transport nutrients to and from wetlands; (2) the water depth; (3) frequency and duration of flooding in tidal marshes |
| marsh peat | the organic soil formed by the accumulation of dead marsh plant material and trapped sediments from tidal waters |
| marsh restoration | improvement of existing marsh condition by reversing some of the adverse impacts caused by coastal development |
| marsh system | an area of marsh associated with a single opening to the ocean, a single freshwater input, or adjacent to and contiguously along the shore of a tidal river or bay |
| mitigation | activities taken to minimize or offset wetland impacts due to development or construction; restoration and enhancement of existing wetlands or creation of new wetlands are forms of impact mitigation |
| non point source | a pollution source that does not come from a single point; typical non-point sources include parking lots, roads, and agricultural fields |
| open water | areas within or adjacent to a marsh that are below mean low water and greater than 100 meters wide (330 feet); this manual uses an arbitrary division of 100 meters to distinguish between open water and tidal creeks |
| organic matter | a combination of decayed and decaying plant and animal residue |
| organic soil | soil consisting of at least 18% organic material (by dry weight) |
| overland flow | a term to describe the sheet-like flow of water over a land surface, not concentrated in individual channels; usually associated with areas of low infiltration such as paved surfaces or surfaces lacking vegetation (see also surface runoff) |
| pannes | shallow ponds that form on the surface of the marsh and hold salt water between tides |
| point source | a pollution source that comes from an identifiable point, such as a factory discharge pipe or septic system outlet |
| primary consumer | animals that eat plant material as their main source of energy |
| primary production | the generation of plant material by photosynthesis |
| riverine | of or pertaining to a river |
| riverine emergent | riverine wetlands dominated by erect rooted herbaceous hydrophytes |
| scrub-shrub | woody vegetation less than 20 feet in height including true shrubs, saplings and trees and shrubs stunted by environmental conditions |
| sea level | the level of the surface of the ocean at its mean (average) position between high and low tide |
| sheet flow | unchannelized flow of water across the surface of a marsh or upland |
| shell middens | a pile of shells remaining from the harvesting of shellfish by Native Americans and early settlers; shell middens are historic relics |
| site specific method | an evaluation method which examines only the subject area without comparison to any other area |
| subsidence | a sinking of the marsh surface, through compaction and degradation of marsh peat; often occurs when Spartina patens is deprived of tidal flow |
| spit | a small point of land, especially sand or gravel, formed by the deposition of material by wind and water currents that runs into a body of water |
| spoils | dredged or excavated soil |
| spring high tide | tides associated with the full and new moon that are higher and lower than the average tide |
| staddle | a structure consisting of numerous pilings driven into the marsh on which to stack salt hay to keep it above the tidewaters until it could be hauled off |
| substrate | the type of bottom sediments such as sand, gravel, peat |
| surface runoff | the movement of water over the land surface (usually in defined channels), resulting from rainfall or snowmelt; percentage of precipitation that becomes runoff varies depending on the slope of the area, the degree of soil saturation, amount of vegetated coverage, or type of surface (e.g., paved areas) |
| tidal amplitude | the variations in the height of tides caused by the lunar cycle, elevation above sea level, the barometric pressure, tidal restrictions and the seasons |
| tidal creeks | streams in the tidal marsh that are less than 100 meters wide at mean low water and whose main source of water is dominated by tidal action |
| tidal flats | areas that are irregularly exposed and are devoid of emergent vegetation, also called mud flats or unconsolidated bottom |
| transition zone | area surrounding a wetland where conditions gradually change from wetland biota to upland biota |
| turbidity | the clarity of the water column as determined by the presence of suspended particles making the water cloudy |
| upland islands | areas of upland soils and vegetation located within a tidal marsh |
| upland peninsula | areas of upland soils and vegetation that extend into the tidal marsh, and are surrounded on three sides by the tidal marsh |
| vegetated tidal marsh | marshes dominated by emergent vegetation and influenced by the tides |
| water column | the habitat that exists in standing or flowing water extending in a column from the surface of the water to the surface of the substrate |
| watershed wetlands | the area from which all water including precipitation, streams and rivers drain to a single point those areas that are inundated or saturated by surface or ground water, support a prevalence of vegetation adapted to life in saturated conditions (i.e., hydrophytes), and are characterized by hydric soils; these include bogs, marshes, swamps, wet meadows, and similar areas |
| Zone of Influence | area surrounding a wetland in which the activities that take place have an impact on the wetland; the Maine Citizens Tidal Marsh Guide considers a 1/2 mile Zone of Influence, with particular focus on activities within the 250 foot shoreland Zone |

Appendix B

REFERENCES

Literature Cited and Additional References Used in the Development of the Manual

Adamus, P.R. and L.T. Stockwell.  1983.  A Method of Wetland Functional Assessment: Volumes I & II Report No. FHWA-1P-82-23 and FHWA-1P-82-24.  Office of Research and Development, Federal Highway Administration, U.S. Department of Transportation, Washington, D.C.

Adamus, P.R. et al.  1987.  Wetlands Evaluation Technique. (WET): Volume II: Methodology, Operational Draft Technical Report Y-87-, U.S. Army Engineer Waterways Experiment Station, Vicksburg, Miss.

Ammann, A.P. and A. Lindley Stone.  1991.  Method for the Comparative Evaluation of Nontidal Wetlands in New Hampshire.  NH Department of Environmental Services, Concord, NH  NHDES-WRD-1991-3

Cook, R.A., A.J.L. Stone, and A.P. Ammann.  1993.  Method for the Evaluation and Inventory of Vegetated Tidal Marshes in New Hampshire (Coastal Method).  Audubon Society of New Hampshire, Concord, NH.

Cowardin, A.P. et al.  1979.  Classification of Wetlands and Deepwater Habitats of the United States.

U.S. Fish and Wildlife Service, Washington, D.C.  FWS/OBS-79/31

Gosselink, J.G., E.P. Odum, and R.M. Pope.  1974.  The Value of Tidal Marshes.  Center for Wetland Resources, Louisiana State University, Baton Rouge.  30 pp.  Sea Grant Publ.  74-03.

Jacobson, H.A., G.L. Jacobson, Jr., and J.T. Kelley.  1987.  Distribution and abundance of tidal marshes along the coast of Maine.  Estuaries.  Vol. 10, No. 2, p. 128-131.  June 1987.

Kelley, J. T., D. F. Belknap, G. L. Jacobson, Jr., and H. A. Jacobson.  1988.  The morphology and origin of salt marshes along the glaciated coastline of Maine, USA.  Journal of Coastal Research.  Vol. 4, No. 4, p. 649-665.  Fall, 1988.

Larson, J. S.  1991.  North America.  Pages 57 - 84 in Finlayson, M. M Moser, editors.  Wetlands.  Facts On File, Inc., New York, NY.

Montgomery, C.W.  1989.  Environmental Geology.  Wm. C. Brown Publishers, Dubuque, Iowa.

Tiner, R.W.  1987. Coastal Wetland Plants of the Northeastern United States.  The University of Massachusetts Press, Amherst, Mass.

Wilson, J.  1993.  Estimated Value of Maine’s Fisheries.  University of Maine, Department of Resource Economics, Orono, ME.

Recommended Reading/Suggested Field Guides

COASTAL WETLAND ECOLOGY

Classification of Wetlands and Deepwater Habitats of the United States. A.P. Cowardin, et al.. U.S. Fish and Wildlife Service, Washington, D.C.  FWS/OBS-79/31. 1979.

Coastal Wetland Plants of the Northeastern United States. R.W.  Tiner.  The University of Massachusetts Press, Amherst, Mass. 1987.

The Ecology of a New England Salt Marsh. M.D. Bertness, American Scientist Vol. 80 pp. 260-268. 1992.

The Ecology of New England High Salt Marsh: A Community Profile. S.W. Nixon, U.S. Fish and Wildlife Service, Office of Biological Services, Washington, D.C.  FWS/OBS-81/55 1982.

The Ecology of Tidal Freshwater Marshes of the United States East Coast:  A Community Profile. W.E Odum,. et al. US Fish & Wildlife Service, Office of Biological Services,Washington, D.C.  FWS/OBS-83/17 1984.

The Ecology of the Great Bay Estuary, Maine and Maine: An Estuarine Profile and Bibliography. F.T.  Short. National Oceanic and Atmospheric Administration, Washington, D.C. Coastal Ocean Program Publication 1992. ed.

Life and Death of the Salt Marsh. J. Teal, M. Teal. Ballantine Books, New York, N.Y. 1969.

The Wetlands of Acadia National Park and Vicinity.  A.J.K. Calhoun, et al.  University of Maine, Orono, National Park Service, U.S. Fish and Wildlife Service.  Maine Agricultural Experiment Station Miscellaneous Publication 721.

Wetlands. W.J. Mitsch., J.G. Gosselink. Van Nostrand Reinhold, New York, N.Y. 1986.

Wetlands: The Audubon Society Nature Guide. W.A. Niering,. Alfred A. Knopf Inc., New York, N.Y. 1988.

Coastal Wetland Planning and Management

Coastal Management Techniques, A Handbook for Local Officials, Land & Water Associates and Maine Tomorrow, Hallowell, ME, Maine Dept. of Economic and Community Development, Augusta, ME, October 1988.

The Estuary Book, Maine Coastal Program, Maine State Planning Office, Augusta, ME.

Financing Marine and Estuarine Programs: A Guide to Resources, Environmental Protection Agency, Washington, DC, September 1988.

Guidelines for Maine’s Growth Management Program, Maine Department of Economic and Community Development, Office of Comprehensive Planning, Augusta, ME, 1988.

Managing Maine’s Harbors and Waterfronts, Maine State Planning Office and Maine Dept. of Economic and Community Development, Office of Comprehensive Planning, Augusta, ME, November 1989.

Managing the Shoreline for Water Dependent Uses, A Handbook of Legal Tools, prepared for the New England/New York Coastal Zone Task Force by the Marine Law Institute, University of Maine School of Law, Portland, ME, December 1988.

Natural Resources Handbook, A Planning Tool for Maine Communities, prepared for the Maine State Planning Office, by the Maine Association of Conservation Commissions, July 1987.

Watershed: An Action Guide to Improving Maine Waters, Maine State Planning Office, Maine Department of Environmental Protection, and University of Maine Cooperative Extension, Augusta, ME, April 1990.

Rtt Bibliography

INVENTORY:

Tidal Crossing Handbook, A Volunteer Guide to Assessing Tidal Restrictions, Timothy A. Purinton and David C. Mountain, Ph.D., Parker River Clean Water Association, 69p.1998(?)

Tidal Crossing Inventory PRCWA

Rim of the gulf : restoring estuaries in the Gulf of Maine. Rockland, Me. : Island Institute, c1998.  145 p.

Maine  Intertidal Habitats, 198 , Maine State Planning Offic

The Geology of Maine’s Coastline, 1983, Maine State Planning Office. 79p

Maine wetlands and their boundaries : a guide for code enforcement officers. Tiner, Ralph W.  Augusta, Me. : Maine Dept. of Economic and Community Development,             1994., 74 p

Maine Citizens Guide to evaluating, Restoring and Managing Tidal Marsh 1997, Robert R. Bryan, Michelle(sic) Dionne, Ph.D, et al. , Maine Audubon Society, 87p

A survey of Impounded Salt Marshes in Southern Maine, Steven Linnell, 199

CMGE maps

NWI maps

Topos

Air Photos

PRIMER:

Guide to Common Marine Organisms along the coast of ME

Well NERR- description/species list

WWW guides-PRCWA

Marblehead

TITLE        The Estuary book : a guide to promoting understanding and

regional management of Maine's estuaries and embayments.

PUBLISHER    Augusta, Me. : Maine Coastal Program, Maine State Planning

Office, 1991.  48 p. : ill., maps

GOR Stacks           GC1021.M2 E78 1991

Appollonio book

Secrets of a salt marsh, Snow John O.,  Portland, Me.,  G. Gannett Pub. Co., 1980, . 76p

AUTHOR       Bertness, Mark D., 1949-

TITLE        The ecology of Atlantic shorelines.

PUBLISHER    Sunderland, Mass. : Sinauer Associates, c1999. 417 p. : ill. (some col.), maps

SUBJECT      Coastal ecology -- Atlantic Coast (U.S.)

ORO stacks           QH104.5.A84 B47 1999

RESTORATION:

Tidal Marshes Of Long Island Sound ,  Ecology, History, And Restoration, Edited By Glenn D. Dreyer And William A. Niering, The Connecticut College Arboretum,  Bulletin No. 34, 1995, 72p.

Ct Arb 12- CT Salt Marshes

Ct Arb 18- CT Seaweeds

Ct Arb 19- CT inland wetland plants

Ct Arb 20- CT Tidal Marsh Invetebrates

Ct Arb 22- peat analysis

Ct Arb 23- Plants and Animals of the estuary

Ct Arb 27- Birds of CT Salt Marshes

living with the shore CT

# Restoring Habitat- map- like LIS NY/CT

Salt Marsh Restoration and Monitoring Guidelines, NY State Dept of State, Division of Coastal Resources (in Press)

Restoration of Coastal Habitats and Species in the Gulf of Maine, Christopher D. Cornelisen, GOM Council on the Marine Environment, 1998

Funding for Habitat Restoration Projects, A citizen’s guide, Restore Americas Estuaries, 1998, 1200 NY Ave NW, suite 400 Washington DC 20005

GENERAL REFERENCES:

The Changing Face of New England, Betty Flanders Thomson,  MacMillan, 1958

URSUS QH104.5 N4T48.

Spartina, John Casey

AUTHOR       Stilgoe, John R., 1949-

TITLE        Alongshore.

PUBLISHER    New Haven : Yale University Press, c1994.

PHYS DESCR   x, 443 p. : ill., maps ; 24 cm.

NOTE         Includes bibliographical references (p. [424]-438) and index.

GOR Stacks           F72 C3 S79

AUTHOR

Siry, Joseph V. (Joseph Vincent), 1949-

TITLE        Marshes of the ocean shore : development of an ecological ethic.

PUBLISHER    College Station : Texas A&M University Press, c1984.

NOTE         Bibliography: p. [193]-205. index.

SUBJECT      Salt marsh ecology.               Coastal zone management.

POR Stacks           Por QH541.5.S24 S57 1984

Review of us marsh policy- starts in 16th cent, extensive references, author based in CA, covers of NE , development of ecology., policy and legislation

TITLE        Applied wetlands science and technology.

PUBLISHER    Boca Raton : Lewis Publishers, 1994

NOTE         Includes bibliographical references and index.

SUBJECT      Wetland conservation. Ecosystem management. Wetlands.  Water quality management.

ALT AUTHOR   Kent, Donald M.

x4 > POR Stacks           QH75 .A44 1994

good set of articles by many authors, good overview, article by Buchsbaum of Mass Aububon on coastal marsh management

NH Method for Evaluation and inventory of vegetated tidal marshes in NH (coastal method), 	Richard A. Cook,

AUTHOR       Davis, Richard A. (Richard Albert), 1937-

TITLE        The evolving coast / Richard A. Davis, Jr.

IMPRINT      New York : Scientific American Library : Distributed by W.H. Freeman and Co., c1994.

231 p. : ill. (some col.), col. maps .

SUBJECT      Coast changes.  Geology.

PPL Main Adult Stacks       551.4 D263 , URSUS  AUG  GB451.2 D39 1994

AUTHOR       Amos, William Hopkins.

TITLE        Atlantic & Gulf coasts.

PUBLISHER    New York : Knopf, [1985]

PHYS DESCR   670 p. : ill. (some col.) ; 22 cm.

SERIES       Audubon Society nature guides.

SUBJECT      Natural history -- Atlantic Coast (U.S.) -- Handbooks, manuals,

ORO stacks           QH104.5.A84 A47 1985

AUTHOR       Eleuterius, Lionel N.

TITLE        Tidal marsh plants.

EDITION      1st Pelican ed.

PUBLISHER    Gretna : Pelican, 1990.

PHYS DESCR   168 p. : ill. (some col.) ; 27 cm.

NOTE         Includes bibliographical references (p. 161) and index.

SUBJECT      Salt marsh plants -- Southern States -- Identification.

ORO stacks           QK125 .E35 1990

AUTHOR       Portnoy, J. W. (John W.)

TITLE        Effects of diking, drainage and seawater restoration on salt marsh biogeochemical cycling.

PUBLISHER    Boston : New England System Support Office, National Park

Service, 1995. PHYS DESCR   vi, 113 p. ill. ; 28 cm.

SERIES       Technical report; NPS/NESO-RNR/NRTR/96-04.

ORO Gov Doc          I 29.109:NPS/NESO-RNR/NRT

AUTHOR       Long, S. P.

TITLE        Saltmarsh ecology.

PUBLISHER    Glasgow : Blackie ; New York : Distributed in the USA by Chapman and Hall, 1983.

PHYS DESCR   viii, 160 p. : ill. ; 21 cm.

ALT AUTHOR   Mason, C. F.

FK Stacks            QH541.5.S24 L66 1983

AUTHOR       Adam, Paul, PhD.

TITLE        Saltmarsh ecology.

PUBLISHER    Cambridge ; New York : Cambridge University Press, 1990.

PHYS DESCR   xii, 461 p. : ill., ; 23 cm.

POR Stacks           QH541.5.S24 A33 1990

AUTHOR       Vadas, R. L.

TITLE        Ecological evaluation of an artifically created salt marsh at

Harrington, Maine : executive summary.

PUBLISHER    [Bangor, Me.] : Dept. of Transportation, Bureau of Highways,

Materials and Research Division, 1986.

PHYS DESCR   16 leaves ; 28 cm.

ALT AUTHOR   Garwood, P. E.

MSL Maine Doc        T 76/1.20:85-5 c.2        AVAILABLE

AUTHOR       Chabreck, R. H.

TITLE        Coastal marshes : ecology and wildlife management.

PUBLISHER    Minneapolis : University of Minnesota Press, c1988.

PHYS DESCR   xiii, 138 p. : ill. ; 24 cm.

SERIES       Wildlife habitats.

POR Stacks           QH104.5.G84 C48

Author from Loiusiana, wildife management oriented,  focuses on Gulfo of Mexico

TITLE        Rim of the gulf : restoring estuaries in the Gulf of Maine.

PUBLISHER    Rockland, Me. : Island Institute, c1998.

PHYS DESCR   145 p. : ill. (some col.), maps (some col.) ; 28 cm.

NOTE         Organizations that contributed authors to this report include the

Island Institute, the Conservation Law Foundation and the

Conservation Council of New Brunswick.

Includes bibliographical references.

MSL Stacks           577.786 R575 1998         AVAILABLE

AUTHOR       Tiner, Ralph W.

TITLE        Maine wetlands and their boundaries : a guide for code

enforcement officers.

PUBLISHER    Augusta, Me. : Maine Dept. of Economic and Community Development,

1994.

PHYS DESCR   ii, 74 p., [8] p. of plates : ill. (some col.) ; 28 cm.

GOR Stacks           QH541.5.M3 T5 1994

AUTHOR       Ursin, Michael J.

TITLE        Life in and around the salt marshes; a handbook of plant and

animal life in and around the temperate Atlantic coastal

marshes,

PUBLISHER    New York, Crowell [1972].

SUBJECT      Salt marsh ecology -- Atlantic Coast (U.S.)

GOR Stacks           QH541.5.S24 U78 1972

Concise, older, good on taxonomy.

Tidal Crossing Handbook, A Volunteer Guide to Assessing Tidal Restrictions, Timothy A. Purinton and David C. Mountain, Ph.D.

Parker River Clean Water Association, 1998(?)

AUTHOR       MacFarlane, Ruth B. Alford.

TITLE        Collecting and preserving plants for science and pleasure.

PUBLISHER    New York : Arco Pub., c1985., 184 p

FK Stacks            QK61 .M32 1985

Good handbook for herbarioum techniques- collection, preserving and mounting, also taxonomy

AUTHOR       Townsend, Charles Wendell, 1859-1934.

TITLE        Sand dunes and salt marshes.

PUBLISHER    Boston : L.C. Page & company, c1925.311 p.

ORO stacks           574 T663

QH104.5.N4 P4 1969

Sea Flavor

Collection of newspaper pieces about Cape and.Ipswich marshes- atmospheric

Daiber

QH75 .D34 1986

Author from U Del- wildife mgt oriented- good literature review including europe

QH104.5.G84 C48 1988

QH75 .M386 1997

SUGGESTED SOURCES OF INFORMATION

| Information on: | Available From: |
| --- | --- |
| Aerial Photographs | James W. Sewall Co. Greater Portland Council of Governments Muncipal Offices |
| Archaelogical and Historic Information | Maine Historic Preservation Commission |
| Endangered and Threatened Wildlife | Maine Department of Inland Fisheries and Wildlife Endangered and Nongame Program U.S. Fish and Wildlife Service Gulf of Maine Project |
| Exemplary Natural Community and Rare Plant Listings | Maine Natural Areas Program |
| Municipal Tax and Zoning Maps | Municipal Offices |
| National Wetland Inventory Maps | Maine Geological Survey Maine Office of GIS (digital information) U.S. Fish and Wildlife Service Gulf of Maine Project 1(800)USA-MAPS www.nwi.fws.gov |
| Protected Lands | Local Land Trusts Municipal Offices Maine Coast Heritage Trust Maine Office of GIS |
| Public Boat Launches | DeLorme Maine Atlas and Gazetteer |
| Restoration | U.S. Fish and Wildlife Service U.S. Army Corps of Engineers Maine Department of Inland Fisheries and Wildlife Maine Department of Environmental Protection |
| Shellfishing Information | Maine Department of Marine Resources |
| USGS Topographic Maps | Maine Geological Survey local bookstores and sporting goods stores 1(800)USA-MAPS |
| Migratory Birds, Anadromous Fish, and Wetland Wildlife Habitat | U.S. Fish and Wildlife Service Gulf of Maine Project, Maine Department of Inland Fisheries and Wildlife Maine Department of Marine Resources |

| Contact Addresses |  |
| --- | --- |
| Greater Portland Council of Governments 233 Oxford Street Portland, ME 04101 (207)774-9891 | James W. Sewall Co. P.O. Box 433 147 Center Street Old Town, ME 04468 (207)827-4456 |
| Maine Audubon Society P.O. Box 6009 118 US Route 1 Falmouth, ME 04105 (207)781-2330 | Maine Coast Heritage Trust 167 Park Row Brunswick, ME 04011 (207)729-7366 |
| Maine Dept. of Environmental Protection State House Station #17 Augusta, ME 04333-0017 (207)287-7688 | Maine Department of Inland Fisheries & Wildlife Endangered and Nongame Wildlife Program 650 State Street Bangor, ME 04401-5609 (207)941-4466 |
| Maine Dept. of Marine Resources State House Station #21 Augusta, ME 04333-0021 (207)287-2291 | Maine Dept. of Transportation State House Station #16 Augusta, ME 04333-0016 (207)287-2841 |
| Maine Office of GIS State House Station #125 Augusta, ME 04333-0125 (207)287-6144 | Maine Geological Survey Dept. of Conservation State House Station #22 Augusta, ME 04333-0022 (207)287-2801 |
| Maine Historic Preservation Commission 55 Capital Street State House Station #65 Augusta, ME 04333 (207)287-2132 | Maine Natural Areas Program Dept. of Conservation State House Station #93 Augusta, ME 04333-0093 (207)287-8044 |
| State Planning Office Maine Coastal Program State House Station #38 Augusta, ME 04333-0038 (207)287-3261 | Wells National Estuarine Research Reserve 342 Laudholm Farm Road Wells, ME 04090 (207)646-4521 |
| U.S. Army Corps of Engineers 424 Trapelo Road Waltham, MA 02254-9149 1(800)343-4789 | Casco Bay Estuary Project 312 Canco Road Portland, ME 04103 (207)879-6300 |
| or |  |
| RR2, Box 1855 Manchester, ME 04351 (207)623-8367 | U.S. Fish and Wildlife Service Gulf of Maine Project 4R Fundy Road Falmouth, ME 04105 (207)781-8364 |
| U.S. Department of Agriculture NRCS 5 Godfrey Drive Orono, ME 04473 | U.S. Fish and Wildlife Service 1033 So. Maine Street Old Town, ME 04468 (207)827-5938 |

Appendix C

U.S. FISH & WILDLIFE SERVICE

WETLAND CLASSIFICATION SYSTEM

In 1979 the U.S. Fish & Wildlife Service (USFWS) published a classification of wetlands and deepwater habitats (Cowardin el al., 1979).  In this classification scheme, wetlands are defined by hydrology, soils, and vegetation.  The USFWS classification scheme serves as the national standard for wetland classification, and has been used to classify wetlands appearing in National Wetlands Inventory (NWI) maps which are used to define marsh systems in the Maine Citizens Tidal Marsh Guide.

The wetland and deepwater habitats of the coastal zone are defined in the USFWS classification as follows:

Wetlands:	Wetlands are lands transitional between terrestrial and aquatic systems where the water table is usually at or near the surface, or the land is covered by shallow water.  For the purposes of the classification, wetlands must have one or more of these three

attributes:	(1) at least periodically, the land must support predominantly hydrophytes (wetland plants); (2) the substrate is predominantly undrained hydric soil; and (3) rocky, gravelly, or sandy areas that are saturated with or covered by shallow water at some time during the growing season.

Deepwater Habitats:  Deepwater habitats include permanently flooded areas deeper then 6.6

feet.  Shallower permanently flooded areas are often vegetated with emergent plants and are considered wetlands rather than deepwater habitats.

The structure of the classification scheme is hierarchical, with systems forming the highest level of the classification hierarchy.  Of the five major systems, three are of interest with regard to tidal waters.

1.	Marine System - Open ocean overlying the continenml shelf including high energy shore line such as beaches and rocky headlands.

2.	Estuarine System - Deepwater and wetland areas that are usually semi-enclosed with an opening to the ocean and in which there is some mixing of fresh and sea water.

3.	Riverine System - Freshwater rivers and their tributaries along with most associated wetlands.

Marine and Estuarine systems are divided into two sub-systems:

1.	Sub-tidal - Areas that are continuously submerged.

2.	Intertidal - Areas that are alternately flooded and exposed.

Riverine systems are divided into four sub-systems, only one of which is relevant to the Maine Citizens Tidal Marsh Guide:

1.	Tidal - the movement of the water is influenced by the tides but water salinity is less than 0.5 ppt.

The next step in the hierarchical system is class.  These classification terms describe the general appearance of the habitat in terms of substrate or the dominant plant community type.

1.	Aquatic Bed - Wetlands that are dominated by plants that grow principally on or below the surface of the water.

2.	Rocky Sbore - Wetlands that are characterized by bedrock, boulders or stones which cover more the 75% of the area.

3.	Unconsolidated Shore - Wetland habitats having three characteristics.  (1) less than 75% coverage by bedrock, boulders, or stones; (2) less than 30% coverage by persistent vegetation; and (3) alternately exposed and flooded.

4.	Unconsolidated Bottom - Wetland habitats having at least 25% cover of particles smaller than stones, and a vegetation cover of less than 30%.

5.	Emergent Wetland - Wetlands dominated by erect, rooted herbaceous hydrophytes.

These wetland classifications should cover any tidal wetland that will be evaluated using the Maine Citizens Tidal Marsh Guide.  Formerly tidal areas that will be included in the inventory may have changed to any one a variety of freshwater systems.   A brief description of some of these systems may help in the identification of these formerly tidal wetlands.

1.	Palustrine System - All non-tidal wetlands dominated by trees, shrubs, and persistent emergent vegetation.

2.	Lacustrine System - Open water wetlands situated in topographic depressions with

less than 30% vegetative cover and greater than 20 acres in size.

Some of the classes that rnay apply to these formerly tidal areas are:

1.	Scrub-shrub - Wetlands dominated by shrubs and tree saplings less than twenty feet in height (e.g., buttonbush, alders and red maple saplings).

2.	Forested Wetland - Wetlands dominated by trees greater than twenty feet in height (e.g., red maple, ash, spruce).

3.	Emergent Wetland - Wetlands dominated by erect, rooted herbaceous hydrophytes.

Also included in the classification scheme are a number of modifiers that are added to the end of the classification abbreviation.  One of these is important in the recognition of formerly tidal areas.  A small “h” signifies that a wetland has been impounded by the purposeful obstruction of flow.

The USFWS wetlands classification system is used as the basis for the wetland identification codes used on the National Wetland Inventory maps.  On the bottom of each NWI map is a key to the complete codes.  The examples below provide examples of some of the wetland classes that will be encountered when using the NWI maps in coastal areas.

E2EM1P	E = Estuarine		E2US4M	E = Estuarine

2 - Intertidal			2 = Intertidal

EM =Emergent		US = Unconsolidated Shore

1 =  Persistent			4 = Organic

P = Irregularly Flooded		M = Irregularly Exposed

E2US3N	E = Estuarine		PUBHh	P = Palustrine

2 = Intertidal			UB = Unconsolidated Bottom

US = Unconsolidated Shore	H =  Permanently Flooded

3 = Mud		h = Diked/Impounded

N = Regularly Flooded

E1UB4 	 E = Estuarine                       	RIUBH          	R = Riverine

1 = Subtidal		1 = Tidal

UB = Unconsolidated Bottom	UB = Unconsolidated Bottom

4 = Organic		H = Permanently Flooded

For a more complete explanation of the classification scheme, the reader may obtain copies of the Classification of Wetlands and Deepwater Habitats of the United States from the US Fish & Wildlife Service.  Reprints of the publication may be purchased from the National Technical Information Service, 5285 Port Royal Road, Springfield, Virginia.

COASTAL WETLAND TYPES

Aquatic Beds: These wetlands form in sub-tidal areas of both marine and estuarine waters.  Along the coast, aquatic beds are areas of seaweed that grow below the low tide level.  In the estuarine waters of the state the most important aquatic beds are eelgrass (Zoster marina) found in protected bays and the major tidal rivers.  Eelgrass beds are important as nursery and feeding areas for fish, as feeding areas for geese, ducks and wading birds, and for trapping and accreting suspended sediments in the water column.  Within some of the larger tidal marshes along the Maine coast aqautic beds of widgeon grass (Ruppia maritima) can add to the diversity of the tidal marshes.

Brackish Marshes:  In areas where average salinities range kom 0.5 ppt. to 18 ppt., a wide variety of plant communities can grow which represent the transition from salt marsh to freshwater marsh.  These marshes can be found along the major tidal rivers and bays and along the smaller freshwater tributaries flowing into salt marshes.  Plants that can be found in brackish areas include black grass (Juncus gerardii), narrow-leaved cattail (Typho angustifolia), and salt marsh bulrush (Scirpus robustus).

Cobble, Gravel, and Sand Beaches:  These are high-energy coastal wetlands formed by the sorting of sediment material moved by wind and wave energy.  The intertidal zone of these wetlands is nearly devoid of visible biota.  The higher reaches of these wetlands, where the wave energy only reaches during storm events, may form sand dunes.  Maine has few remaining dune fields, but all of these areas are presently protected by law.  Dunes support a specialized plant community that is very susceptible to damage during the dune overwash that accompanies large storms.

Freshwater Tidal Marshes:  In areas where the tides still affect the flow of waters but where the average salinity is below 0.5 ppt. freshwater tidal marshes can form.  Vegetation in these marshes is extremely diverse.  In the regularly flooded areas one may find pickerel weed (Pontederia cordata) and wild rice (Zizania aquatica).  In areas that are irregularly flooded  sweet flag (Acorus calamus) and river bulrush (Scirpus fluviatillis) are common.  Freshwater tidal marshes are predominantly associated with the Kennebec River above Bath.

Rocky Shores:  This type of coastal wetland is very common in northern New England.  It can be found in areas where bedrock is exposed by nearly continuous wind and water driven energy.  These wetlands can be divided into three zones:  the salt spray zone - rarely flooded but influenced by waves; the intertidal zone - regularly flooded and exposed by the tides; and the sub-tidal zone - rarely exposed and underwater most of the time.  Plants and animals such as seaweeds, barnacles, and periwinkles can be easily found.

Salt Marshes: These vegetated tidal wetlands, where salinities range from 18 ppt. to 34 ppt. (the latter is that of seawater), are dominated by Spartina grasses.  In low marsh areas that are flooded twice daily, saltwater cordgrass (Spartina alterniflora) forms nearly mono-specific stands that vary in height from a few inches to five feet in height.  On the high marsh salt meadow grass (Spartina patens) is the dominant plant, but it is usually found in association with numerous other plants that can tolerate high salinity levels (halophytes).

Tidal or Mud Flats:  These wetlands are unvegetated, low relief environiments particularly common in protected coastal environments.  They are of critical importance for the production of numerous invertebrate species which are a food source for many bird and fish species.  When flooded, the mud flats are scoured by fish feeding on the worm and mollusk population found in the muddy substrate.  As the tide recedes, wading birds feed on the same food source.  Mudflats can also be found in the larger tidal marshes providing diverse habitat within the marsh.

Appendix D

BENEFITS AND DETRIMENTS

OF SALT MARSH PLANTS AND WEEDS

# BENEFITS OF NATIVE SALT MARSH PLANTS

Native salt marsh plants benefit ecosystems in many ways:

- Submersed plants pump oxygen into the water for animals such as fish.

- Aquatic and salt marsh plants provide hiding places and nurseries for animals.

- Aquatic and salt marsh plants provide surface area for small organisms such as algae and shrimp that are often eaten by larger organisms.

- Aquatic and salt marsh plant parts are eaten by fish, waterbirds, and other animals.

- Aquatic and wetland plants absorb excess nutrients and other forms of pollution.

DETRIMENTS OF WEEDS

## Non-native and native aquatic and wetland plants can become “weeds” in alien habitats:

- Aquatic and wetland weeds can replace native plants, depriving native animals of their natural habitats and food supplies.

- Aquatic and wetland weeds can cover water bodies and wetlands, preventing oxygenation and “suffocating” fish and other animals.

- Aquatic and wetland weeds can prevent the use of water bodies for fishing, swimming, and other recreation.

- Aquatic and wetland weeds can disrupt navigation by both small and large vessels.

- Aquatic and wetland weeds can greatly slow or even stop water flow, disrupting flood control and irrigation systems.

WHAT IS A GENUS AND WHAT IS A SPECIES?

Taxonomy is the scientific classification of organisms.  Plant and animal taxonomy is arranged in a hierarchy, from phylum down to species.

Phylum or Division

Class

Order

Family

Genus

# Species

A species is a group of individuals that can successfully interbreed with individuals of the same species, producing fertile offspring.  A genus is a group of closely related species.

In scientific names, the first word is the genus name, and the second word species name.  Thus, in the example, Typha latifolia, Typha is the genus name, and Typha latifolia is the species name.

The term, Typha spp., refers to all species in the Typha genus.

DEFINITION OF TERMS FOR PLANT IDENTIFICATION

Here are several descriptions that must be remembered if you really want to be able to identify plants, particularly the grasses, sedges and rushes.  There are many other parts and terms of course, but here we describe only the terms used for basic plant identification.

Some leaves have sheaths.  The sheath is at the base of the leafblade and it may completely surround the stem, or it may not.  The sheath may be short or it may be very long.

Another part is the ligule.  The ligule is an outgrowth at the junction of the leaf blade and the leaf sheath.  The ligule may be hairy, or bristly, or hard or soft or may be absent altogether.

An inflorescence is a structure that holds the flowers of a plant.  There are different types of inflorescences having various degrees of complexity.  Often, inflorescences are described as being closed or opened.  The main axis of an inflorescence is called a rachis.  The rachis may have a few branches, or many branches.  The branches may be further divided into branchlets.

The inflorescence may have structures called spikelets.  Spikelets are parts that contain the flowers of the plant.  Spikelets may grow in rows, or in spherical clusters, or in other arrangements.  Depending on the species, spikelets may be extremely small, or quite large, an inch or more long.  Spikelets often have scales and/or bristles.

The base of the inflorescence or its branches may have bracts.  Bracts are leaf-like parts that may be very small and inconspicuous, or they may be relatively large or long, in which case they may be mistaken for leaves.

The individual flower of grasses, sedges and rushes are usually not conspicuous and colorful like the flowers of other plants.  In fact, non-botanists often don’t recognize them as flowers at all, even when magnified.  The flowers may be so tiny that they require a magnifier to see, such as Juncus.  Or they may be so numerous that they are very obvious, such as Phragmites.

# IDENTIFICATION OF GRASSES, SEDGES AND RUSHES

Many people have difficulty separating grasses, sedges and rushes from one another.  However, the most basic and helpful way to remember these different types of salt marsh plants is by this catchy phrase:

“Sedges have edges, rushes are round, grasses have joints when the cops aren’t around!”

The following are more biological terms for distinguishing the three different groups.

Grasses:  Grasses have jointed, hollow stems that are round in cross-section.  Leaves are distinctly two-ranked and connected to the stem by open sheaths.  Leaves possess a ligule (membranous or hairy appendage) at the junction of the leaf and the leaf sheath.  Flowers are born in spikelets, and each flower consists of two glumes (bracts) and one or more florets (each of which has two different bracts, the lemma and the palea, and a flower).  Fruits are grainlike seeds.

Sedges:  Sedges have solid, triangular edges for the most part.  Leaves are three-ranked with closed sheaths.  Flowers are born in the axils of overlapping scales, which may collectively resemble a bud.  Each flower consists of a single pistil with two to three stigmas and one to three stamens.  Fruits are lens-shaped or three angled nutlets called achenes.

Rushes:  Rushes have solid stems, mostly round in cross-section.  No ligule is present.  They have regular flowers with three sepals, three petals, three or six stamens, and a fruit capsule.  Fruit capsules are usually three-halved but sometimes one-celled.  Rushes can be easily separated from grasses and sedges by their multiseeded capsules, because grasses and sedges have only one seed per flowering scale.

# IDENTIFICATION OF INVASIVE PLANT SPECIES

Within a salt marsh, invasive plants often thrive in areas of reduced salinity or high nutrient levels.  Thus, they are often the symptom of restricted tidal flows and/or other human influences on the marsh system, such as input of nutrients from runoff and septic systems or improperly installed culverts.  The objective of this section is to outline the types of invasive species that may be found in some of the salt marshes you inventory.  The presence of these species are a good indication that a restriction is occurring.  Combining this information with an inventory and evaluation of tidal restrictions will help to prioritize sites for more detailed monitoring and possible restoration.

The target species of the search will be common reed (Phragmites australis) and purple loosestrife (Lythrum salicaria).  Volunteers should also record the presence of narrow-leaf cattail (Typha angustifolia) and broad-leaved cattail (Typha latifolia), as extensive growth of these species may indicate the effects of a tidal restriction or nutrient source.

Common reed (Phragmites australis)

Tall perennial grass up to 14 feet high often forming dense stands, with long tapered conspicuously two-ranked (up to 24 inches long and 2 inches wide), and dense much-branched purplish to brownish terminal inflorescence (8 to 16 inches long); July through September; salt and brackish marshes and freshwater tidal and nontidal marshes.

Purple Loosestrife (Lythrum salicaria)

Medium height to tall perennial herb up to 6 feet high with angled almost woody stems, entire lance-shaped leaves often having heart-shaped bases, sometimes in whorls of three, and many pink to purplish five to six petaled flowers borne in leafy spikelike inflorescence (up to 16 inches long); July through September; inland marshes, wet meadows, shrub swamps, tidal fresh marshes, and upper edges of coastal marshes.

Narrow-leaved Cattail (Typha angustifolia)

Tall grasslike perennial herb up to 10 feet high with narrow leaves (to ½ inch wide) and inconspicuous flowers borne in terminal spike (persistent through the winter) composed of two separate parts (male flowers above and female flowers below): May to July; coastal marshes mostly in brackish waters, but also along edges of salt marshes and inland marshes.-

Broad-leaved Cattail (Typha latifolia)

Has wider leaves (up to one inch wide) and a continuous terminal spike separated into two distinct parts.

SALT AND BRACKISH MARSH PLANTS OF MAINE

The following list of plants contains those species found in Maine salt and brackish marshes.  This list should be used in conjunction with Coastal Wetland Plants of Northeastern United States by Ralph Tiner to help in the identification of plant species needed to complete the Maine Citizens Tidal Marsh Guide.

| galinis maritima | Seaside Gerardia |
| --- | --- |
| Amaranthus scanabinus | Water Hemp |
| Ammophila breviligulata | Marramor Beachgrass |
| Arenaira peploides | Seabeach Sandwort |
| Artemesia stelleriana | Dusty Miller |
| Artemisia caudata | Tall Wormwood |
| Aster subulatus | Annual Salt Marsh Aster |
| Aster tenuifolius | Perennial Salt Marsh Aster |
| Atriplex glabriuscula | Orach |
| Atriplex patula | Orach |
| Bassia hirsuta | Hairy Smotherweed |
| Cakile edentula | Sea-Rocket |
| Carex scoparia | Pointed Broom Sedge |
| Carex hormathodes | Marsh Straw Sedge |
| Cladium mariscoides | Twig-Rush |
| Distichlis spicata | Spike Grass |
| Eleocharis halophila | Salt Marsh Spike-Rush |
| Eleocharis parvula | Dwarf Spike-Rush |
| Eleocharis smallii | Small’s Spike-Rush |
| Elymus virginicus | Virginia Rye Grass |
| Euphorbia polygonifolia | Seaside Spurge |
| Glaux maritima | Sea Milkwort |
| Hordeum jubatum | Squirrel-Tail Grass |
| Hudsonia tomentosa | Beach Heather |
| Iva frutescens | Marsh Elder of High-Tide Bush |
| Juncus balticus | Baltic Rush |
| Juncus canadensis | Canada Rush |
| Juncus gerardii | Black Grass |
| Juncis greenei | Green’s Rush |
| Lathyrus japonicus | Beach Pea |
| Leachea maritima | Pinweed |
| Limonium nashii | Sea Lavender or Marsh Rosemary |
| Lythrum salicaria | Purple Loosestrife |
| Myrica pensylvanica | Northern Bayberry |
| Panicum virgatum | Switchgrass |
| Phragmites australis | Common Reed |
| Pinus rigida | Pitch Pine |
| Plantago maritima | Seaside Plantain |
| Polygonella articulata | Sand Jointweed |
| Polygonum aviculare | Common Knotgrass |
| Polygonum ramosissium | Bushy Knotweed |
| Potamogeton pectinatus | Sago Pondweed |
| Potentialla anserina | Silverweed |
| Prunus maritima | Beach Plum |
| Puccinellia maritima | Seaside Alkali Grass |
| Quercus alba | White Oak |
| Quercus bicolor | Swamp White Oak |
| Ranunculus cymbalaria | Seaside Crowfoot |
| Rosa palustris | Swamp Rose |
| Rosa rugosa | Salt Spray Rose or Rugosa Rose |
| Rosa virginiana | Virginia Rose |
| Ruppia maritima | Dtich or Widgeon Grass |
| Salicornia bigelovii | Bigelow’s Glasswort |
| Salicornia europaea | Common Glasswort or Samphire |
| Salicornia virginica | Perennial or Woody Glasswort |
| Sanguisorba canadensis | Canadian Burnet |
| Scirpus acutus | Hard-Stemmed Bulrush |
| Scirpus americanus | Three-Square |
| Scirpus atrovirens | Green Bulrush |
| Scirpus cyperinus | Wool Grass |
| Scirpus maritimus | Salt Marsh Bulrush |
| Scirpus paludosus | Bayonet-Grass |
| Scirpus robustus | Salt Marsh Bulrush |
| Scirpus validus | Gerater Soft-Stemmed Bulrush |
| Smilax rotundifolia | Common Greenbriar |
| Solidago sempervirens | Seaside Goldenrod |
| Spartina alterniflora | Saltwater Cordgrass |
| Spartina patens | Salt Meadow Grass |
| Spartina pectinata | Freshwater Cordgrass or Slough Grass |
| Spergularia canadensis | Canada Sand Spurrey |
| Spergularia marina | Salt Marsh Sand Spurrey |
| Suaeda linearis | Sea Blite |
| Suaeda maritima | Sea Blite |
| Suaeda richii | Sea Blite |
| Toxicodendron radicans | Poison Ivy |
| Triglochin maritima | Seaside Arrow Grass |
| Typha angustifolia | Narrow-Leaved Cattail |
| Typha latifolia | Borad-Leaved or Common Cattail |
| Zannichellia palustris | Horned Pondweed |
| Zostera marina | Eelgrass |

Appendix E

# MAP REFERENCE NUMBER

The Map Reference Number is constructed from the Town and estuary abbreviations below plus a sequential number for each crossing in each town.  Example:  Scitterygusset Creek/Lunt Road Crossing in Falmouth is F/PR 2 – Falmouth/Presumpscot River second crossing in Falmouth (generally numbers are assigned from south and or west to north and/or east, moving downeast along the coast and clockwise around bays)

| TOWNS AND CITIES: CE- Cape Elizabeth SP- South Portland P-Portland LI- Long Island F-Falmouth C-Cumberland Y- Yarmouth F-Freeport BR- Brunswick H- Harpswell WB- West Bath P- Phippsburg | TOPO QUAD NAMES: PE- Portland East PW- Portland West Y- Yarmouth F- Freeport SH- South Harpswell BR- Brunswick OI- Orrs Island BI- Bailey Island BA- Bath PH- Phippsburg SP- Small Point |
| --- | --- |

TOPO QUAD NAMES:

PE- Portland East

PW- Portland West

Y- Yarmouth

F- Freeport

SH- South Harpswell

BR- Brunswick

OI- Orrs Island

BI- Bailey Island

BA-  Bath

PH- Phippsburg

SP- Small Point

Appendix F

CASCO BAY TIDAL MARSH CROSSING LISTING

Listing compiled from inspection of USGS topo maps- from Portland HeadLight to Small Point roughly from south and west to north and east. List includes all crossings of streams below first contour. Info supplemented from DeLorme atlas, nautical chart and Champion Map

M=mapped, D=Data Sheet,*=to be examined, II=Phase II data/ map ref number/ road name/ watercourse or nearest bay /remark

# V=Void- deleted from Data Base

CASCO BAY DRAINAGE

PORTLAND WEST QUAD

SOUTH PORTLAND

MD	SP/FR 1		Cottage Rd/Mill Creek

MD	SP/FR/9		S. Portland walkway (s edge of Mill Creek Park)/ Tributary of Mill Creek – marsh

MD	SP/FR/10	S. Portland walkway (s edge of Mill Creek Park)/ Mill Creek – marsh

MD	SP/FR 2		Broadway/Anthoine Creek

M  *	SP/FR 3		Public walkway at Henry St. (just e of Turner I) pond above

MD	SP/FR 4		Northeast Petroleum Terminal Causeway/Barberry Creek

V	SP/FR 5		West edge of Mobil Terminal next to Forest City Cemetery- no marsh

M  *	SP/FR 6		Mobil (rolling Mills) -Calvary Cemetery

V	SP/FR 7		Clark Pond Dam VOID-DAM

V	SP/FR 8		Long Creek w Branch? - Gravel pits VOID-NO CROSSING

MD	SP/FR 12	Kelsey St/Anthoine Creek

MD	SP/FR 11	Kelley St/Anthoine Creek

PORTLAND

V	P/FR 1		Stroudwater Dam VOID-DAM

`M   *	P/FR 2		Fore River-Congress St. Bridge

MDII	P/FR 3		N Division RR/Capisic Brook

MD	P/FR 15		N Division RR/Fore River Marsh, Capisic Brook, West of Maine Channel

V	P/FR 4		Creek trib to & w of Capisic along RR fill VOID – NO CROSSING

M *	P/FR 5		N Div RR/Trib Fore R w of Thompsons pt- (off end of Powsland Street)

MDII	P/BC 6		Baxter Blvd./Cheverus HS

MDII	P/BC 7		Baxter Blvd./Fall Brook

MDII	P/BC 8		Baxter Blvd./ Payson Park

MD	P/PR 9		I-295/marsh- N Deering –southern of 2

MD	P/PR 11		I-295/marsh- N Deering –northern of 2

M  *	P/PR 10		Casco Ironworks site

PORTLAND E QUAD

FALMOUTH

M  *	F/PR 		I-295 just north of Falmouth Line

M	MF/PR 1	I- 295/trib of Presumpscot

MD	F/PR 2		Lunt Rd/Scitterygusset

M *	F/FS 3		Pond @ Portland CC/dam??

M  *	F/FS 4		RT 88/Mill Creek Falmouth msa

M  *	F/FS 5		US RT 1/ Mill Creek

LONG ISLAND

M  *	LI/CB 1		Fern Ave./ Fowler Beach E end

M  *	LI/CB 2		Fowler Beach W end

M  *	LI/CB 3		Harbor Grace/Fern Ave

PORTLAND

V	P/CB 12		Peaks I: Seashore Ave./stream nr Elm Tree Cove- ice pond VOID-ABOVE TIDE

M  *	P/CB 12		Great Diamond  I./ ice pond at Diamond Cove

M  *	P/CB 14		Cushing I Diane Nolan??

YARMOUTH QUAD:

YARMOUTH

V	Y/RR 1		VOID

M  *II	Y/RR 2		Prince Point Rd/ stream off Broad Cove nmsa

MD	Y/RR 3		Treatment plant Rd/tributary of Royal River

M  *	Y/RR 4		Town Landing Rd/Trib of Royal R?

M  *II	Y/RR 5		Old County Rd/Pratts Brook

M  *	Y/RR 6		Granite St – I-295 /Pratts Brook  restricted channel?

FREEPORT

MD	F/RR 11		Old County Rd/Cousins R Trib running N  (stream between Pratts & Harvey Brooks)

MD	F/RR 8		Old County Rd/Cousins R

MD	F/RR 9		Webster Rd/ Harvey Brook

MD	F/RR 10		Me Central RR e of Webster Rd/ Merrill Brook

FREEPORT QUAD

M  *	F/HR 1		S Freeport Rd/trib off Spar Cove

M  *	F/HR 2		S Freeport Rd/Porter Landing Pond drain

M  *	F/HR 12		Old Dam/stream running N from Porter Landing

M  *	F/HR 3		Torrey Range Rd/ trib off Porter ldg

M  *	F/HR 4		Bow St./Frost Gully Brook  (AKA Pump house Brook) repl Oct 99

M  *	F/HR 5		Bow St./Mill stream repl Oct 98

MD	F/CB 6		Burnett Rd/ Little R

BRUNSWICK

V	BR/MA 7		Flying Pt Rd/Bunganuc Stream VOID- ABOVE TIDE

SOUTH HARPSWELL QUAD

no crossings

ORRS I QUAD

V	BR   10		VOID

MD	BR/MA 11	Maquoit Rd/ stream e of Wharton Pt

MD	BR/MA 12	Rossmore Rd/stream at head of Maquoit Bay

M  *	BR/MI 13	road bet Merepoint and Simpson Pt /Miller Cr (PRIVATE)

M  *	BR/MI 14	RT 123 S from Dyers corner/stream of Middle Bay- new pond on topo?

MD	BR/MI 15 	Harpswell Neck Rd/stream at head of Middle Bay (w of 2)

MD	BR/MI 16	Harpswell Neck Rd/stream at head of Middle Bay (e of 2) dam? Pond

MD	BR/HS 19	Prince Rd /Buttermilk Cove

MD	BR/HS 17	New Gurnet Rd / stream running N from head of Buttermilk Cove

M   *	BR/HS 18	Road running E from Dyer Corner to New Gurnet Rd/ Mare Brook (private-NAS)

HARPSWELL

M   *	H/HS 4 		Rt. 123/neck at Skolfield cove to Middle Bay- new pond?

M   *	H/HS 16		Road off  Rt. 123 to High Head/stream at head of Mill Cove

MD	H/HS 2		Doughty Cove Rd.  W of Rt. 24/ stream e of Misery Hill

MD II	H/HS 3		Long Reach Lane W of Rt. 24 to Dyer Cove/Doughty Cove

MD	H/QB 1		rd E of Rt 24 to Tondreaus PT/stream at head of Orrs Cove

MD	H/QB 5		Rt. 24/stream fm Dyer Cove(e shore)

MD	H/HS 8		Rt. 24/pond S of  Reed Cove (dam)”Wilson’s Pond”

MD	H/HS 9		dam on E side of Gun Pt-“Dan’s Pond”/Gun Pt Creek

V	H/      6		VOID

MD	H/HS 10 	Shore Rd off Cundy’s Harbor Rd just N of Cranberryhorn Cem/ stream S off “Laurel Cove”  W of N end of Long I.

M	H/NM 21       	Shore Rd off Cundy's Hbr Rd /N Branch of Stream flowing into New Meadows

M	H/NM 22  	Wallace Shore Rd E off Cundy Hbr Rd /Stream flowing S into Quahog Bay

M	H/NM 23  	Wallace Shore Rd E off Cundy Hbr Rd /N Branch of Stream flowing into N Meadows

MD	H/NM 11  	Wallace Shore Rd E off Cundy Hbr Rd Stream flowing into N Meadows

MD	H/HS 12		Grover Lane off Cundy Hbr Rd E to loop (new, not on Topo)/ stream flowing S into Quahog Bay

M	H/HS 17		Mountain Road/	Strawberry Creek

M	H/HS 18		head of Widgeon Cove- new road across marsh?

M     	H/QB 24	Cundy Hbr Rd/stream just TO Quahog Bay from Cove SE of Rich Cove

MD	H/QB 23	Bethel  Pt rd/stream running s fm Quahog Bay off Tide Mill Cove- big marsh

MD	H/QB 13     	Bethel Pt Rd/pond off Hen cove

MD	H/QB 14     	dam just w of Bethel; Pt- marsh

M	H/NM 15	E Cundy Pt Road- marsh off Sandy Pt Cove

H/ HS/7		Henry Creek Way off Mountain Road/Henry’s Creek  running S from Long Reach- bridge, unrestricted

BAILEY I QUAD

H/HS 19		Marsh a S end of Lowell’s Cove on W side of Bailey I N of  Mackerel

H/CB 20		new Road/ stream n off Ash Cove

BRUNSWICK QUAD

M  *	BR/NM 1	New Gurnet Rd/Mare Brook (private-NAS)

M  *	BR/NM 2	Road & RR on NAS/ Mare Brook (private-NAS)

V	BR/NM 3	Board Rd/Head of Woodward Cove –VOID- ABOVE TIDE

MD	BR/NM 4 	Adams Rd/head of Thomas Bay

V	BR/NM 5	Bath Rd/head of Thomas Bay- VOID -ABOVE TIDE

V	BR/NM 6	RR/head of Thomas Bay VOID -ABOVE TIDE

SMALL POINT QUAD

PHIPPSBURG

MD	PH/SP13	Route 216/ stream N from eastern arm of Cape Small Harbor, head of marsh no culvert

M  *	PH/SP 14	Flat Pt Rd/ W branch of  stream N from eastern arm of Cape Small Harbor

M  *	PH/SP 1		Flat Pt Rd/ stream N Small Pt Harbor, Bailey Beach? E of Flat Pt (W of 2)

M  *	PH/SP 12	Flat Pt Rd/stream N from middle arm of Cape Small Harbor (E of 2)

M	PH/SP 2		Alliquippa Rd w of  Rt 216  toward Goose Rock/ E branch of stream N fm middle arm of Cape Small Hbr -msa

M   *	PH/SP 11	Alliquippa Rd w of  Rt 216  toward Goose Rock/ stream N fm west arm of Cape Small Hbr -msa

V	PH       3		VOID

MD  II	PH/SP 4		RT 216 (Small Pt Rd)/ Eastern arm of  Cape Small Hbr msa-

MD	PH/SP 5		Head beach Rd- beach

MD	PH/SP 6		Rd to lookout tower @ cem/ outlet of Big Pond msa

PHIPPSBURG QUAD

MD	PH/NM 8	Basin Rd/ head of Wah Tuh Pond nmsa

MD	PH/NM 10	Pine ?  Rd  at Sebasco Resort/ outlet of  Wah Tuh Pond- brackish? fresh

MD	PH/NM  9	North Creek off Tottman Cove- numerous xing msa – all above tide

BATH QUAD

Brunswick

MD	BR/NM 8	Bath Rd/ Marsh west of New Meadow River  running NE- no culvert?

M  *	BR/NM  9	Marsh Between Bath Rd & RR tracks ?drain E along S side of RR tracks to New Meadows R.

West Bath

MD  II	WB/NM 1	Bath Rd/New Meadows R at Marina/New Meadows Inn

M	WB/NM 2	RR/ Mew Meadows R

M	WB/NM 3	Rt. 1/ New Meadows R

MD	WB/NM 4	Old Brunswick Rd (topo says “old Bath Rd”)/ New Meadows R msa

V	WB/NM 5	VOID

MD	WB/NM 6	Hill Rd/head of Mill Cove W- high

MD	WB/NM 7	Berrys Mill Rd/ Eastern head of Mill Cove- pond – perched brackish

MD	WB/NM 8	Road E off Berry Mill Rd/ to WJTO Radio Tower stream  above pond easterly off  Dam Cove Creek nmsa

MD	WB/NM 	Berry Mill Rd/ W of 2 Streams running N off Dam Cove- fresh, no marsh

WB/NM 9		Berry Mill Rd/Westernmost of 2 streams running N off  Dam Cove Creek, fresh, no marsh

KENNEBEC DRAINAGE

Bath

Ba/ken/ 1 		/Winnegance Cr

PH/Ken			/Cutting Cr

Ph/ken/15		Center Pond

Appendix G

Casco Bay Return the Tides Project: Tidal Crossing Data Sheet Phase 0

| Location |  |  |  |
| --- | --- | --- | --- |
| Map Reference Number |  | Unique number: (assigned by GIS) |  |
| Town |  | Topo Quad name |  |
| Water Body/Stream Name |  |  |  |
| Street |  |  |  |
| Landmark/Location Description |  |  |  |

Estuary

Lat/Lon of Crossing (GPS/GIS)

Topo info

NWI info

CMGE info

Air photo number

Air photo info

Road jurisdiction

Marsh ownership

Marsh Area (GIs)

Priority for study

Comments

Appendix H

# RETURN THE TIDES PROJECT: Tidal Crossing Data Sheet- Phase I Part 1

| Location |  |  |  |
| --- | --- | --- | --- |
| Map Reference Number (Town/Estuary/sequential no.) |  | Unique no. (assigned by GIS) |  |
| Town |  | Top Quad Name |  |
| Water Body/Stream Name (From Topo, if not named, get local info) |  |  |  |
| Street |  |  |  |
| Landmark/Location Description |  |  |  |

| Field visit Information |  |
| --- | --- |
| Date/time/volunteer name(s) |  |
| Weather | Sunny Partly Cloudy Overcast Rain |
| Tide | Incoming Outgoing Change/Slack |
| A Photo #1 - Reference #: |  |
| B Photo #2- Reference #: |  |
| C Photo #3- Reference #: |  |
| D Photo #4- Reference #: |  |
| E Photo #5- Reference #: |  |

| Crossing Information | Remarks |
| --- | --- |
| Type a) Bridge Culvert Other (describe in detail) b) Road on Marsh Headland to Headland c) Road runoff: Onto Marsh Into Creek |  |
| Opening: Approx. dimensions. record shape and material of crossing structure |  |
| Nature of crossing: Upstream Downstream | Perched culvert, pooling, scour, bank slumping, rubble in stream Perched culvert, pooling, scour, bank slumping, rubble in stream |
| Upstream Channel Width at Crossing | Distance from Crossing: |
| Downstream Channel Width at Crossing | Distance from Crossing: |
| Length of Crossing over marsh |  |
| Approx. Crossing Width (In Middle) | Culvert length (bottom)/Bridge width (abutments) |
| Road Description | 1 2 3 4 lanes curbs paved sidewalks |
| Road Surface Material | Asphalt Concrete Gravel Dirt Other (describe) |
| Condition (1-5*) of Bridge | 1 2 3 4 5 |
| Condition (1-5*) of Road | 1 2 3 4 5 |
| Condition (1-5*) of Culvert | 1 2 3 4 5 |

*1. Excellent 2. Good 3. Fair 4. Poor 5. Need of Immediate Repair

PHASE I , PART 1 DATA SHEET  (continued)

MAP REF #:

Circle the Most Appropriate Response

|  | Restriction Classification Scheme |  |
| --- | --- | --- |
| Classification | Evidence of Flow Restriction/Erosion | Notes |
| 1 Upstream / Downstream | Unrestricted/ No Pooling |  |
| 2 Upstream / Downstream | Flow Detained/ Slight Erosion |  |
| 3 Upstream / Downstream | Minor Pooling/Erosion Present |  |
| 4 Upstream / Downstream | Significant Pooling/Significant Erosion Present |  |
| 5 Upstream / Downstream | Major Pooling/Major Erosion Present |  |
| Classification | Channel vs. Culvert /Opening |  |
| 1 Upstream / Downstream | River Width < Opening Width |  |
| 2 Upstream / Downstream | River Width = Opening Width |  |
| 3 Upstream / Downstream | River Width 1.1 to 2.0x Opening Width |  |
| 4 Upstream / Downstream | River Width 2.1 to 5.0x Opening Width |  |
| 5 Upstream / Downstream | River Width 5.1x + Opening Width |  |
| Classification | Vegetation Comparison |  |
| 1 | Upstream = Downstream |  |
| 2 | Upstream Slightly Different Than Downstream |  |
| 3 | Upstream Different From Downstream |  |
| 4 | Upstream Much Different Than Downstream |  |
| 5 | Upstream Completely Different Than Downstream |  |
| Overall Rating = |  |  |

|  | Flood Potential | Notes: |
| --- | --- | --- |
| 1-High | Structures likely to be flooded if restriction removed |  |
| 2- Med | Need detailed survey to determine flooding risk |  |
| 3- Low | Flooding unlikely |  |

Narrative Description and General Notes: (attach additional pages as needed)

Volunteer signature(s):

Casco Bay Return the Tides

# Instructions for Tidal Crossing Data Sheet, Phase I, Part 1

Equipment:  Field map, Phase One data sheets (one per crossing), blank paper, RTT fact sheets, pencils, measuring tape, rubber boots or mud sneakers, camera, film, clipboard, orange vest.

Part 1 is a preliminary assessment of the crossing to record its basic characteristics and determine if it is apparently restrictive.  If, after completing Part 1, you determine that the crossing is apparently restrictive, proceed to gather the more detailed Part 2 information.  The most restrictive crossings will be selected for Phase II tidal curve measurements.  For Part 1, all dimensions should  be estimated by pacing or armspan in the interest of time. These dimensions will be measured more precisely if the crossing is apparently restrictive and gets the Part 2 assessment.

Data points:

# Map reference number:  Town/estuary/sequential number by town.

Unique number- leave blank, the GIS program assigns this number automatically when the record is entered in the system

Town name:  Town, city or other minor civil division.

Topo Quad name:  use abbreviation of topographic quadrandgle   name from map reference list.

Water body/stream name:  From topo map or from local knowledge/information if not named on topo map (indicate if unmapped local name).

Street:  Name of the street crossing water body/stream.  If not a street, insert other identification such as “Guilford Rail line.”

Landmark/Location Description:  For small unmarked streams with multiple crossings, record distance and direction from a unique mapped landmark on the topo map (e.g., 0.6 miles east of US HWY 1 on Blue Point Road).

FIELD VISIT INFORMATION:

Tidal Conditions:  Circle approximate tide height (low, mid, etc.) and tidal current flow direction (incoming, outgoing, change/slack).

## Photographs:

## #1,2: Standing on the creek bank, photograph the crossing from the upland 	(“upstream”) and seaward (“downstream”) sides.

## #3,4: Standing at the crossing, photograph the marsh looking upstream and 	downstream.

## #5: Finally take at least one overview photograph of the marsh and crossing to show the 	entire setting, this may be from a nearby high point where the road emerges onto the 	marsh.

## Instructions for Tidal Crossing Data Sheet, Phase I, Part 1, page 2

## Record a description of  the locations where the photos were taken.  Record roll and photo 	number, mark and label printed photos.  Example:  “SP/FR  2: Anthoine Creek /Broadway.  	View from upland side”.

Crossing type:  Indicate Bridge/Culvert/Other as defined.

BRIDGE - A structure that spans a stream between abutments on each side and which does not have a hard bottom.  The stream flow and sediment load defines the height of the stream bottom under a bridge.  Some structures that look like bridges actually are culverts because there is a solid bottom for the channel under the structure.

CULVERT - A structure to pass water under a road or other crossing which has a bottom as well as sides and a top.  A culvert’s bottom, unless it is set sufficiently deep so that the stream bed establishes equilibrium above the culvert bottom, artificially raises the stream bed height and restricts flow at lower tide levels.  Culverts are commonly made of round or oval pipe, or of concrete, often in box section.  One or more pipes or boxes may be installed at a single crossing. Record shape, size and construction material for each culvert forming the crossing.

OTHER- If the crossing is neither a culvert nor a bridge, (anything else such as a dam, flume, spillway, fish ladder, ditch, etc.) describe it in detail and 	sketch it.

Road on Marsh:  Does the road approach the crossing on fill placed on the marsh surface, or does the crossing go from headland to headland (no fill on marsh)?  Circle appropriate choice.

Road Runoff:  Note and describe how road runoff is directed at the crossing and approaches. Freshwater runoff directed onto the marsh can damage it.

Dimensions of opening:  Record height and width of opening (smallest dimension) or diameter if round.  If  there are multiple culverts, note number of culverts and the dimensions of each.  Also note material: for culverts:CIP- corrugated iron pipe, iron pipe,  corrugated plastic pipe, concrete pipe, concrete box etc. For bridges, note the clear opening width and height (to the lowest point on the bridge or other objects such as pipes under the bridge.

Upstream/Downstream Channel Width:  Restrictive bridges and culverts often cause substantial erosion of stream banks, especially near the restriction.  Measure the channel width at a point beyond any erosion that may be due to increased current speed through the restriction.  Except for very small creeks (less than 10’ wide), fifty feet from the crossing is a minimum distance.  Note the distances, upstream and downstream, from the crossing where the measurements were taken and indicate location on sketch.  If possible, measure at equal distances upstream and downstream from the crossing.

Defining channel width:  Many tidal creeks have gently sloping banks, making “channel width” hard to define.  For this study, measure channel width from the high marsh/low marsh dividing line on each side of the creek.  This is typically the upper limit of Spartina alterniflora. Note any difficulties in measuring width.

Nature of Crossing:   note and circle  all applicable features.

Instructions for Tidal Crossing Data Sheet, Phase I, Part 1, page 3

Length of crossing over marsh:  Estimate the length of the crossing from upland to upland along the centerline of the road or other feature.

Crossing width:  Estimate the entire width of the fill on the marsh surface immediately adjacent to the crossing.

Culvert length/Bridge width:  Pace off the total length of the crossing along the centerline of the stream and record approximate width.

Road description:  Circle all applicable road features.

Road Surface Material:  Note type of material such as asphalt, concrete, gravel, dirt, other (describe)

RESTRICTION CLASSIFICATION SCHEME

Evidence of Flow Restriction/Erosion:  As you visually assess the crossing, characterize the bank erosion and pooling and circle an appropriate classification for upstream and downstream.

As with any ranking approach, objectivity is difficult to maintain, especially when first starting.  When ranking the degree of bank erosion, the lowest scores should be given to those sites that most closely resemble natural stream conditions.  This resemblance can be determined by studying the surrounding downstream conditions and similar streams that are not affected by road crossings.  As you observe more, you will become familiar with typical conditions.  Highly eroded sites will show bank failure or slumping; wide, rounded creek pools; and the build-up of rubble and riprap in the streambed.

Channel vs. Culvert Opening:  Use the dimensions recorded above to compute the crossing ratio and circle an appropriate classification for upstream and downstream.

Vegetation Comparison:  Analyzing the differences in vegetation from upstream to downstream can take a trained eye, especially since the differences may be subtle. Moreover, the differences may not be related to a restriction of salt water.  Visually assess the habitat both upstream and downstream and record any difference in frequency of salt-tolerant and salt-intolerant plant such as common reed (Phragmites australis) or freshwater species such as cattails (Typha sp.) and common loosestrife (Lythrum Salicaria).  In extreme cases, the habitat may evolve into shrub or forested swamp, and the former wetland may be invaded by upland species.  Circle appropriate classification for upstream and downstream areas.  Refer to the plant handouts for additional information.

Rank flood potential as follows:

High- 		High probability that structures near the marsh will be flooded if the restriction is removed.

Medium- 	The potential for induced flooding can be ascertained only through detailed engineering field surveys.

Low-	There are no structures located near the marsh or the structures present are so high above the marsh that induced flooding   is highly unlikely

Instructions for Tidal Crossing Data Sheet, Phase I, Part 1, page 4

# Narrative comments: Write a very brief narrative description of the crossing, the associated marsh and surroundings. Comment on any special features not apparent from the data recorded above which might make this crossing or marsh a candidate for closer study or restoration.  Historic information and local knowledge about the site, its use (bird watching, hunting, harvesting, launching, etc.) and local value may be significant.

# APPENDIX I

# RETURN THE TIDES PROJECT:  Phase I Tidal Crossing Data Sheet, Part 2

| Location |  |  |
| --- | --- | --- |
| Map Reference Number (Town/Estuary/sequential no.) |  |  |
| Town |  |  |
| Water Body/Stream Name (From Topo, if not named, get local info) |  |  |
| Field visit Information | (mark N/A if same as Part 1) |  |
| Date/time/volunteer name(s) |  |  |
| Weather | Sunny Partly Cloudy Overcast Rain |  |
| Tide | Incoming Outgoing Change/Slack |  |

Plan View Sketch

1”= North Approximate Scale (Feet)

Volunteer  Signature(s):

PHASE I , PART 2 DATA SHEET  (continued)

MAP REF #:

| Crossing Information (MEASURED) | Remarks |
| --- | --- |
| Dimensions of Opening: Height, Width, Diameter (if Applicable) Include Sketch. Shape and material of crossing structure |  |
| Upstream Channel Width at Crossing | Distance from Crossing: |
| Downstream Channel Width at Crossing | Distance from Crossing: |
| Height of Creek Bottom to Culvert Bottom, Upstream |  |
| Height of Creek Bottom to Culvert Bottom, Downstream |  |
| Height of Creek Bottom to Road, Upstream |  |
| Height of Creek Bottom to Road, Downstream |  |
| Length of Crossing over marsh |  |
| Crossing Width (In Middle) | Culvert length (bottom)/Bridge width (abutments) |
| Road Surface Width (In Middle) |  |

General Notes: (attach additional pages as needed)

# Casco Bay Return the Tides

# Instructions for Tidal Crossing Data Sheet, Phase I, Part 2

# Part 2 is a more detailed assessment of crossings that the Part 1 assessment indicated as apparently restrictive.  Because these crossings are of greater concern, information is collected in more detail and dimensions are measured rather than estimated.

# Location:  Insert Map reference number, Town name and Water body/stream names from Part 1 data sheet.  Also, insert Field visit information if different from Part 1.

# Sketch:  Make a "bird's eye view" sketch of the site, with notable features labeled and upstream and downstream sides indicated.  Indicate which direction is north and the approximate scale of the sketch.  The sketch should show road, fill, crossing, scour pools, marsh, upland, road drainage and incorporate buildings, vegetation types, and crossing configuration.

Dimensions of opening:  Record height and width of opening (smallest dimension).  If multiple culverts, note number of culverts and the dimensions of each.

Upstream/Downstream Channel Width:  Restrictive bridges and culverts often cause substantial erosion of stream banks, especially near the restriction.  Measure the channel width at a point beyond any erosion that may be due to increased current speed through the restriction.  Except for very small creeks (less than 10’ wide), fifty feet from the crossing is a minimum distance.  Note the distances, upstream and downstream, from the crossing where the measurements were taken and indicate location on sketch.  If possible, measure at equal distances upstream and downstream from the crossing.

Defining channel width:  Many tidal creeks have gently sloping banks, making “channel width” hard to define.  For this study, measure channel width from the high marsh/low marsh dividing line on each side of the creek.  This is typically the upper limit of Spartina alterniflora.  Note any difficulties in measuring width.

Height from creek bottom to culvert bottom:  Culverts with bottoms that are above the natural level of the creek bottom may cause tidal delay and pooling.  Record the height of the culvert bottom from the creek bottom.

Length of crossing over marsh:  Measure the length of the crossing from upland to upland along the centerline of the road or other feature.

Crossing width:  Measure the entire width of the fill on the marsh surface immediately adjacent to the crossing.  For busy roads estimate this measurement and note if estimated.

Culvert length/Bridge width:  Measure the total length of the crossing along the centerline of the stream.

Road surface width:  Measure the width of the paved or traveled road surface.

### Instructions for Tidal Crossing Data Sheet, Phase I, Part, page 2

Narrative comments:  Write a brief narrative description of the crossing, the associated marsh and surroundings.  Try to capture the essence of the site, considering the following factors as they may be applicable:  overall value and appeal of the site or extent of degradation (“a little gem of a marsh in an unexpected spot,” “a sodden, stinking mess of putrid trash”), aesthetic value from a wildlife (birds, fish, shellfish, mammals) or landscape perspective (invasive plants, viewshed, etc.), public visibility, educational potential, nature of the surrounding area, potential for restoration and benefits of restoration, potential issues relating to restoration and anything else that may be of interest or helpful in describing the site and its restoration potential.  Historic information and local knowledge about the site, its use (bird watching, hunting, harvesting, launching, etc.) and local value is very helpful.

# APPENDIX J

# RETURN THE TIDES PROJECT: Tidal Crossing Data Sheet- Phase II

## REFER TO INSTRUCTION SHEET FOR DETAILED INSTRUCTIONS

| Location | (copy from Phase I data sheet) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Map Reference Number |  | Unique number: (assigned by GIS) |  |  |  |
| Town |  |  |  |  |  |
| Water Body/Stream Name |  |  |  |  |  |
| Street |  |  |  |  |  |
| Landmark/Location Description |  |  |  |  |  |

| Field visit Information |  |
| --- | --- |
| Date/time/volunteer name(s) |  |
| Weather (circle applicable terms) | Sunny Partly Cloudy Overcast Rain |
| Tide (from tide table) | High Low: |

| Water Levels : Time (Hours: Minutes) | Measure approx every 2 Height Upstream (Inches) | hours Height Downstream (Inches) |
| --- | --- | --- |

Sketch of Reference Points in Relation to Site

1”= ‘ North approximate scale (feet) 1”= North Approximate Scale (Feet)

General Notes (continue on back):

# Casco Bay Return the Tides

# Instructions for Tidal Crossing Data Sheet, Phase II

# Phase II is a detailed measurement of the tidal heights at crossings that the Part I assessment indicated to be apparently restrictive.   It is important to get a set of measurements spread over 12 hours so, in order to do all the measurements in daylight you will need to start about 7 AM!

Field equipment: Phase 2 data sheet with site info, location map, Phase 1 data sheet, Return the Tides Fact sheets, clip board, sharp pencils, weighted tape measure, yardstick, bug juice, sturdy shoes

DETAILED INSTRUCTIONS FOR COMPLETING DATA SHEETS (see sample attached):

# Location: If necessary, insert Map Reference Number, Town name and Water body/stream names from Part 1 data sheet, also insert Field visit information, note unusual weather conditions such as storm, heavy rain, or strong wind which might affect water level.

## Determining Reference Points: Unless reference points have been pre marked, reference points need to be defined on both the upstream and the downstream side of the bridge or culvert. The reference points should be chosen near the middle of the channel, to ensure that your measurements will catch the low water. Reference points should be marked with a small spot of paint or colored chalk. It is important to mark the reference point clearly, for each successive measurement should be performed from the exact same point. A weighted tape measure or a carpenter's tape (for smaller sites) should be used to measure the distance between the water surface and the reference point for both upstream and downstream reference points at each crossing.

# Sketch: Referring to the Phase 1 Sketch, make a  simple "bird's eye view" sketch of the site, Showing the reference points used to measure the tidal heights, include enough detail so that the reference points can be relocated on a later occasion.

# Measuring the Tide: The rise and fall of the tide is measured as the distance between the water surface and your reference points. The measurements need only be to the nearest inch, A total of 6 measurements timed approximately 2 hours apart need to be collected during one day. Timing your 6 measurements at 2-hour intervals allows you to record an entire tide cycle. It is not important that the measurements are taken exactly 2 hours apart, as 15 or 20 minutes either way will not effect the computation of the tidal range. It is important to note the exact time at which each measurement is taken, however. If there is not water at the crossing, measure to the bottom of the channel and describe where the water is in a note- such as “ no water on downstream side, water is out of sight across tide flats”

THINGS TO REMEMBER

Collect complete and ACCURATE data – it’s better to abort readings than to collect inaccurate data

Measure to nearest inch

Read measurements directly off tape

Record exact time of reading

Reference points need not be at same height

Measure with the bottom of the tape just touching the water (making ripples on surface)

Clean out trash to get a clear water surface

Describe any problems in a detailed comment on your data sheet; also include any other observations or comments about the site and your measurements that may be helpful.

When you are finished, please make copies of your data sheets and mail the original sheets to me in the envelope provided in your package. Save the copies in case the originals get lost in the mail.